Appendix 6

Specification of Width, Height and Color of Characters for

Vessel Markings

I. Chinese Vessel Name

| 1. Height Standard of Character |  |  |
| --- | --- | --- |
| GT of Vessel |  | Minimum Length |
| 20 GT and above and less than 100 GT |  | 25 cm |
| 100 GT and above and less than 1,000 GT |  | 30 cm |
| 1,000 GT and above |  | 35 cm |
| 2. Color of Characters |  |  |
| Ground Color of Fishing Vessel | Color of Characters |  |
| Dark Color | White |  |
| Bright Color | Black |  |

II. English Vessel Name and CT Number

| 1. Height Standard of Character |  |  |
| --- | --- | --- |
| GT of Vessel |  | Minimum Length |
| 20 GT and above and less than 100 GT |  | 15 cm |
| 100 GT and above and less than 1,000 GT |  | 20 cm |
| 1,000 GT and above |  | 25 cm |
| 2. Color of Characters |  |  |
| Ground Color of Fishing Vessel | Color of Characters |  |
| Dark Color | White |  |
| Bright Color | Black |  |

III. IRCS

| - Height Standard of Character |  |  |  |  |
| --- | --- | --- | --- | --- |
| Position of Marking | Vessel Length Overall |  |  | Minimum Length |
| Both Sides of the Vessel | 25 meters and above |  |  | 1 meter |
|  | 20 meters and above and less than 25 meters |  |  | 80 cm |
|  | 15 meters and above and less than 20 meters |  |  | 60 cm |
|  | 12 meters and above and less than 15 meters |  |  | 40 cm |
|  | 5 meters and above and less than 12 meters |  |  | 30 cm |
|  | Less than 5 meters |  |  | 10 cm |
| Deck | 5 meters and above |  |  | 30 cm |
| - Width Standard |  |  |  |  |
| Marking Items |  |  | The Proportion of Marking Items to the Height |  |
| Width of Hyphen |  |  | Half |  |
| Stroke Width of Characters or Hyphen |  |  | One-sixth |  |
| Space between Characters |  |  | One-sixth and above and less than one-fouth |  |
| Adjacent Space of Characters with Hypotenuse Form (such as Ａ,Ｖ） |  |  | One-tenth and above and less than one-eighth |  |
| - Color of Characters |  |  |  |  |
| Ground Color of Fishing Vessel |  | Color of Characters |  |  |
| Dark Color |  | White |  |  |
| Bright Color |  | Black |  |  |